G56 NOISE WARNING’S & FACTS

Gear rollover facts:

When converting a G56 transmission over to a solid mass flywheel vs. dual mass flywheel, gear rollover is inevitable. Gear rollover noise is often heard when the vehicle is idling or under a load in low RPM ranges (often referred to as sounding like marbles in the transmission). This is completely normal.

Gear rollover noise is caused by reverse engine spike and can in most cases be drastically reduced by simply changing the oil in the transmission or reducing engine spike produced by inadequate engine components (ex. misfiring injector, FCA problems, Electronic programming). The oil recommended by the transmission manufacture Mercedes-Benz is “Mobil Delvac Synthetic Transmission Fluid 50.” We at Valair Inc. recommend draining completely and refilling with 7 quarts. Only 6 quarts will fit when utilizing the fill hole, the final quart will need to be filled through the shift tower.

Disengagement noise:

On all dual and triple disc clutches, a noise will be associated with the disengagement of the clutch. This is due to the intermediate plate(s) floating freely in the lugs of the flywheel. This is NOT a bad throw out bearing!!! The noise can possibly get more noticeable during the break-in period of the clutch due to the intermediate plate(s) seating in.

Please educate yourself before purchase:

Please research G56 gear rollover and/or transmission related noises due to solid flywheel replacement. Any noise’s caused by gear rollover or rattle from the floater plate will not be covered under any type of warranty, nor can the clutch be returned after install. If you choose to return the clutch prior to installation, a 10% restocking fee will be added to any returns.

Thank you for your cooperation,

Valair Inc. Tech support